RASPBERRY PI PROJECT REPORT

on

Smart Surveillance Camera

By

Jaydeep Chauhan (60944)

# Acknowledgment

The satisfaction that accompanies the successful completion of any task would be incomplete without mentioning the people who make it possible. We are grateful to a number of individuals whose professional guidance along with encouragement, has made it a very pleasant endeavor to undertake this project report. We are truly grateful to Mr. Spundflasch for his valuable guidance and encouragement. His encouraging words went a long way in providing patience and perseverance, which were needed to complete this project. We would like to express our gratitude to Benedikt Bieberle and Robert Schwartz. Finally, we express our sincere thanks to our parents and all those who helped us directly or indirectly in many ways in the completion of this project.

# Abstract

Security concerns govern a big part of people’s daily lives in the current world. That is why we see outrage and wide-reaching concern at incidents like Yahoo breach of 2013 with 3 billion accounts compromised, Facebook-Cambridge Analytica scandal and Quora in 2018 with over 50 million accounts compromised. Security in the real world is a bit easier to achieve, and a simple surveillance camera can accomplish a lot for the average man.

The aim of this project is to design a Surveillance System based on a Raspberry Pi 3. The goal is to monitor the activity of an intended location. Once a human is detected in the region, the system automatically sends alerts to the owner, keeps a tab of the duration of the visit, takes pictures and allows for the live streaming of the security feed remotely.

Keywords: Security Camera, Object Detection, Remote Streaming

# Table of Contents

Acknowledgment	1

Abstract	2

Table of Contents	3

1. Project Idea Based on Task	5

Project Idea	5

Motivation	5

The solution to a real-world problem	5

Existing Examples	5

Personal Skills	6

2. Target Categories	6

Mandatory Goals	6

Target Goals	6

Discretionary Goals	6

3. User Friendliness and Usability	7

Design and Characteristics	7

Problems in the Long Term	8

Solution	8

4. Hardware and Software Components	9

Hardware	9

Software	9

Cloud Services	9

5. Project Structure Plan	10

Division in Subareas	10

Detection	11

Alert System	11

Quality of Life Features	11

Structure plan	11

6. Project execution planning explained	13

Detection	13

Alert System	13

Quality of Life Features	13

7. Challenges         	14

8. Conclusion         	15

9. References	16

10. Project directory Structure:	17

11. Code	18

On BOOT Bash Scripts:	18

dateTimeScript.sh	18

on_reboot.sh :	18

my_script.sh :	18

Project code	19

main.py :	19

camera.py :	19

mail.py :	19

dropboxStorage.py:	19

SpreadSheet.py :	19

TwilioAmazon.py :	20

twilionotifier.py :	20

Index.html :	20

# 1. Project Idea Based on Task

## Project Idea

To design a surveillance system using Raspberry Pi which includes:

  - Recording pictures and videos.

  - Keeping a log of events that occurred.

  - Sending alerts to the owner in case of an event in the form of SMS or email.

  - Making a timelapse video.

  - Streaming live video from a remote server.

## Motivation

Security has become an indispensable need in the modern world. New mechanisms are constantly being developed for adding security to both the internet and the physical world. Security cameras are a basic tool that most people use as a first measure, be it for private use, in stores, businesses or by the police. One drawback, however, is that these are sometimes quite expensive. In this project, we will recreate the simple idea behind a security camera and put it on a cheap platform like a raspberry pi. While a security camera is a common project to be built on a raspberry pi, we decided to improve upon that. To make it a better product, we are developing several features that work together to improve quality-of-life (QoL) as well as add more functionality. Instead of a deep goal, we are spreading the project broader to increase value and complexity.

## The solution to a real-world problem

Wired systems cost less for parts but more for installation, at $150-$200 per camera or even higher in some cases, compared to their wireless counterparts and this figure changes country to country. The installation cost of CCTV packages starts out from $70 to $340 per camera based on the additional features, like high definition image quality and larger digital storage. As such, this might sometimes be too expensive for personal use. For that, a raspberry pi based surveillance camera can be a cheap alternative which can be created within €50-60.

## Existing Examples

Security cameras are one of the most widely used equipment one can find. They are used everywhere from schools, homes, offices, stores, traffic cameras. With this project, we take that widely used idea and put it on a cheap platform. The result is an affordable security system anyone can make with the parts and some technical skills.

## Personal Skills

There are a lot of personal skills and competencies that will play among the group members during the implementation of this project and they are:-  the ability to work in harmony with group members, technical competency, communication skills, flexibility, problem-solving skills, and programming skills.

# 2. Target Categories

## Mandatory Goals

The following objectives are the base of our project. The project is not complete without the fulfillment of these goals:

  - Taking images and videos while monitoring the objects in the frame.

  - Object detection and detection of an event.

  - Keeping a log of events.

  - Sending an alert to the owner through SMS and email upon the detection of an event.

  - Saving images and videos to the cloud.

  - Live streaming the feed from the camera from any network.

## Target Goals

These are our main targets to achieve which make our project stand out:

  - Creating a timelapse of the video feed on demand.

## Discretionary Goals

There are a few things which we would love to add in our project which is given below:

- Limiting detection to humans, so that the system does not consider pets as security threat events.

- Adding the option to save data on multiple cloud servers.

# 3. User Friendliness and Usability

## Design and Characteristics

First, the user has to start the raspberry pi by connecting it to the power source. We assume that peripherals are already connected properly and checked for any loose connection. At the start, the pi automatically configures the date and time, after which we sync the Google logs with the correct time. The program checks if the spreadsheet has a log of the current month and then the current day. If no sheet is found, we create a new spreadsheet file for the current month and in it, we create pages equal to the number of days in that month and in each page we make the necessary columns (Date, File Name, File Type, Location of the file, URL of the file location). If there is one available, we append the day’s logs at the end on that page. Then the main program is automatically executed, and servers are created. We create an SSH server to expose the local host to the public domain via tunneling, which can be accessed remotely.

The pi camera will start generating live feeds and will capture one picture per 15 seconds with timestamp even if there is no motion detected. These pictures will be later used for creating a Timelapse video for the full day.

If the camera detects any human movement, a picture will be clicked marking the moving object (Human in our case) and the camera starts to record video for the next 5 minutes and uploads it to the Amazon S3 bucket. The picture is then uploaded in DropBox for future reference. Details regarding the picture will be saved in the Google spreadsheet. After these three tasks are completed, an email alert is generated for the user which includes a link to the picture taken by the camera. After this, an automatic message will be generated from the Twilio server stating that some Unusual activity is detected and will also provide the URL of the video uploaded in Amazon S3 bucket. The user will then be able to click on the URL in message and watch the recorded video directly from Amazon S3 bucket or start watching a live video stream by typing in access URL generated by ngrok or other streaming services.

## Problems in the Long Term

We tried out all possible ways to keep the budget and complexity of the project under control. We are using trial accounts for storing data in the cloud or to send SMS through Twilio trial account. But in the long run, the user might get space issues as data size might increase drastically depending upon motion detected by the camera. Raspberry Pi might get overheated if used for a long duration, due to which processing speed might decrease. We will be able to monitor a small area with a single camera. The user has to use a dynamic URL address for the live streaming of video after every restart of Raspberry Pi because we are using the ngrok trial account. Under the trial account of ngrok live streaming service, streaming URL gets changed on every restart of the service and it is not possible to control it in a trial account. If there is a power outage, then it obviously doesn’t work, and it has to be started again. Further, the raspberry pi has low computational power and can lag.

## Solution

We think in the future we can solve all these issues with the solutions listed below:

- Use a wide-angle  lens camera to cover a large area

- We can purchase the subscription of cloud accounts (Twilio, AWS S3, DropBox) to avoid space issues.

- User can purchase the subscription of ngrok to use static domain address to overcome the URL change issue.

- The use of heat sink or fan along with proper ventilation can avoid overheating the Raspberry Pi board up to a certain extent.

- Trying to automate the program to better counter power outages, and be more efficient.

# 4. Hardware and Software Components

We will be designing the surveillance camera using:

## Hardware

  - Raspberry Pi 3

  - Pi night vision camera module

  - 3D printed Surveillance camera body

  - Any mobile/laptop/Tablet supporting browser and active Internet connection

## Software

  - OpenCV

  - Flask

  - Ngrok (or Serveo/Pagekite for static URL)

  - Python

## Cloud Services

  - Ngrok port forwarding / Serveo / Pagekite

  - AWS S3 service

  - Twilio SMS service

  - DropBox

  - Google Drive (for Spreadsheet)

# 5. Project Structure Plan

## Division in Subareas

The project is divided into the following sub-parts according to functionality/features:

Fig 1: Project plan

### Detection

  - Event detection: (kw 17-19,24)

  - Saving images at events: (kw 18,19,24)

  - Streaming live video from a localhost: (kw 18,19,24)

### Alert System

  - Email alert: (kw 25)

  - Message alert: (kw 26)

  - Saving data on cloud: (kw 27)

  - Streaming video from any network: (kw 27-28)

### Quality of Life Features

  - Keeping a log of events in the spreadsheet: (kw 27-28)

  - Creating a timelapse video: (kw 28-29)

## Structure plan

Fig 2: Block diagram of the structure plan

# 6. Project execution planning explained

We have broken the project down into 3 portions:

## Detection

- Event detection: It is the most basic part of the project where the camera records the room and detects when an event is occurring like someone coming in. We work on this during weeks 17-19 and 25,26.

- Saving pictures: Alongside monitoring for events, we also take snaps at regular intervals and save these images. This is worked on parallelly with event detection. Worked on in weeks 18,19,25-27.

- Streaming to localhost: The video from the camera can be streamed when desired. This is worked on in weeks 27 and 28.

## Alert System

- Email and SMS alert: At event detection, we send alerts to the owner via emails or messages. These two can be done parallelly after exams in weeks 32 and 33.

- Saving data on the cloud: We complete this portion after alerts are completed, in weeks 34 and 35.

- Streaming video from a remote server: We improve upon our feature of streaming, this is done in weeks 35-37.

## Quality of Life Features

- Keeping a log of events: We keep logs of event detections in a spreadsheet file, this is done in weeks 38 and 39.

- Time Lapse video: We make a timelapse video at the end of the day with pictures taken throughout the day. This is the final feature that will be done in weeks 39-41.

After this is over, we will work on finalizing everything and try to present the project during the weeks 42-44.

# 7. Challenges

There are a few foreseen challenges like how to design the body of surveillance camera to fit raspberry-pi 3 inside it. How to procure internet SMS service provider API and phone number. What should be the size of the SD card used in Raspberry Pi to meet all processing and storage requirements? How to integrate APIs of dropbox and Amazon AWS S3 service to store data in the cloud. How to provide live video feed from any network using port forwarding. How to click pictures when the camera is busy recording the video.

In order to overcome these challenges, we have to do brainstorming with team members and needs to do research over the different alternative solutions.

Technical and programming skills play a major role in the project development cycle.

The following steps were taken to solve the problems listed above and in section 3B:

- We started the early procurement of hardware like the Raspberry Pi board and camera module, and we 3D printed a body for this system with the help of UniKAT.

- We started creating an account in the cloud (Twilio, AWS, Dropbox and Google Drive) and do the necessary permission setting so that there should not be an issue in uploading data from the Raspberry Pi.

- We started developing each function independently and then we integrated that functionality in the main program.

- We programmed a bash script that automatically configures the date and time on the raspberry pi and following this executes the main program. This makes sure that the security system is up and running as soon as the raspberry pi has power. This improves efficiency and is better suited to handle power outages.

- We make use of Serveo instead of ngrok which gives us a static URL for live streaming. While Servio performed very well, it has been temporarily shut down since December 2019 due to phishing activities. As an alternative, we are currently using Pagekite which also provides a static URL but the performance isn’t as good and the service is laggy.

# 8. Conclusion

We have successfully developed a Surveillance system, and we have been able to implement all of the functionality and QoL features that we set out to implement. Our system can monitor the location in its field of view and it detects motion and recognizes when a human has entered the frame, though it can be a little slow. We take pictures throughout the day and store this data along with the timestamps. When an event is triggered, we send the user emails and SMS alerts about the event, following which they can use the URL from the alerts to check the images from the camera or a video.  And on top of that, the user can see Timelapse videos starting at any time of that day, or stream the live feed remotely. All of the event activities are stored in a Google spreadsheet and the log is meticulously maintained every day of every month. Further, the entire operation is automated and all the user needs to do is make sure the system is connected to a power source and that there is an internet source that can be used. The functioning of the system is automated and the program starts as soon as there is power, which is also advantageous in case of power outages as the user does not have to manually start the system afterward. All in all, we have been successful in implementing all the functionality but there is still room for improvements and more features that can be added. Since most of the system is self-assembled, we have even managed to keep the cost down significantly compared to commercial options.

# 9. References

[1]. Raspberry Pi Documentation https://www.raspberrypi.org/

[2]. A. Mansell, "Getting Started with Raspberry Pi: Installing Raspbian," Raspberry Pi, 14 January 2014.

[3]. Source on security camera installation cost: https://www.dksecurity.net/security-camera-installation-cost/

[4]. The Inquirer, "Raspberry Pi Model B+ review," The Inquirer, 30 July 214: http://www.theinquirer.net/inquirer/review/2357623/raspberry-pi-model-b-review.

[5]. Pi setup and basic guides https://www.pyimagesearch.com/

[6]. Security camera example with ngrok https://www.hackster.io/hackershack/smart-security-camera-90d7bd

[7]. Ngrok port forwarding documentation https://ngrok.com/

[8]. Pagekite documentation http://pagekite.net/

[9]. Serveo documentation http://serveo.net/

[10]. Integrating Google Sheets with Raspberry Pi  https://developers.google.com/sheets/

[11]. Amazon AWS service setup guide for storing videos from Raspberry Pi to S3 bucket https://docs.aws.amazon.com/quickstarts/latest/s3backup/step-1-create-bucket.html

[12]. Dropbox integration with Raspberry Pi https://www.dropbox.com/developers/apps/create

[13]. Flask API for creating SMTP servers https://flask.palletsprojects.com/en/1.1.x/api/

[14.] Twilio API for SMS service https://www.twilio.com/

# 10. Project directory Structure:

.

. ├── pi-reboot │ ├── dateTime.log │ ├── dateTimeScript.sh │ ├── my.log │ ├── my_script.sh │ └── on_reboot.sh └──── SecurityCamera ├── alerttools │ ├── notifications │ │ ├── __pycache__ │ │ ├── __init__.py │ │ └── twilionotifier.py │ ├── __pycache__ │ ├── utils │ │ ├── __pycache__ │ │ ├── conf.py │ │ └── __init__.py │ └── __init__.py ├── config │ ├── config.json │ └── dropboxconf.json ├── dbsecurecamera │ ├── __pycache__ │ ├── __init__.py │ ├── tempimage.py ├── models │ ├── facial_recognition_model.xml │ ├── fullbody_recognition_model.xml │ └── upperbody_recognition_model.xml ├── ngrok │ └── ngrok ├── __pycache__ ├── templates │ ├── index.html ├── tempTl ├── timelapse ├── camera.py ├── creds.json ├── dropboxStorage.py ├── housekeeping.py ├── mail.py ├── main.py ├── requirements.txt ├── SpreadSheet.py └── TwilioAmazon.py

# 11. Code

## On BOOT Bash Scripts:

These bash scripts are created and placed under pi-reboot directory, these scripts are executed automatically when raspberry-pi boots up.

### dateTimeScript.sh

This script is called in /etc/rc.local file in raspberry pi at boot time of raspberry pi.

The below script tries to connect to the internet to fetch current DAte and time to update the raspberry-pi.Once date-time is in sync with online time, bash /home/pi/pi-reboot/on_reboot.sh > /home/pi/pi-reboot/my.log command is called to execute another bash script on_reboot.sh

### on_reboot.sh :

As mentioned above, this script is called inside dateTimeScript.sh bash script. This script will first activate the virtual environment with following commands:
		source /home/pi/.profile 
		workon cv

After this, it will execute the python file main.py with the following command:
		python /home/pi/SecurityCamera/main.py

main.py is the main file of our project inside which other python files( have different functionalities) are imported

### my_script.sh :

To execute my_script.sh file, we created a service my_script.service at location /etc/systemd/system/my_script.service and is executed at boot time once the raspberry-pi is successfully connected to the internet. Below is the code for my_script.service

Purpose of the script my_script.sh is to expose the local server in public domain using ssh (in case of Serveo/Pagekite) so that live feed can be streamed from any network by typing some static URL set by the user. Below is the code for my_script.sh :

## Project code

### main.py :

main.py is the main file of our project which is automatically colled at boot time and this file setup connection with Dropbox and SMTP server along with other cloud services.

### camera.py :

This file is used to initialize the camera object to be used in the main.py file. Camera object uses the imutils library to initialize the piCamera module. Any camera resolution changes can be made in the imutils library.

### mail.py :

This file initializes the Flask libraries to create an SMTP server and to send an email alert to the owner. All Gmail authentications and account detail is stored/hardcoded in this file. User should refer Gmail guidelines to create an alternate password using the following link: https://support.google.com/accounts/answer/185833?hl=en

### dropboxStorage.py:

This python file is initialized in the main.py file itself when pi boots up. This file sets up the connection with the DropBox application to store pictures captured later in this project. It will also load the Dropbox configuration files dropboxconf.json inside the config directory.

### SpreadSheet.py :

This file is used to store the media log in the Google spreadsheet. First of all, this file will authenticate the surveillance camera user with the Google drive account. Once authentication is successfully done, it will check if there is already a Google sheet created for the current month or not. If it finds that no Spreadsheet is available for the current month then, it will generate one for the current month and will add sheets for each date of the current month after checking the Leap year condition.
	Once the Sheets are created for each date, then it will first insert-header with the following fields: 'Date','File Name', "File Type", "File stored on", "File URL".After this, it will insert the logs for each media (image, video or Time-lapse video) in the same sheet of the spreadsheet.

On the other hand, if the spreadsheet is already available for the current month, then it will open the sheet for today’s date and will check if there is already some log data in that sheet or not. If no data, then it will first insert-header with following fields: 'Date','File Name', "File Type", "File stored on", "File URL".After this, it will insert the logs for each media (image, video or Time-lapse video) in the same sheet of the spreadsheet. But, if there is some data already present in today’s date sheet, then it will keep on appending the log data without inserting headers.

Users can directly identify the media and the even at which the media is captured by looking at the “File Type” header values. He/She can directly access the media by directly clicking on the link in the "File URL" field of the spreadsheet.

### TwilioAmazon.py :

This file is used to create the Twilio object tn to send alert messages. It will also load the Twilio configuration files config.json inside the config directory.

### twilionotifier.py :

This file has all the functionality to trigger the SMS alert mechanism in the Twilio server. Twilio account authentication is also done in the same file. Apart from SMS functionality, this file is also used to upload the videos in the amazon AWS S3 bucket after authenticating the user account. Link to access this video is sent to the owner via Twilio SMS.

### Index.html :

Index.html file is the webpage created to be accessed in port #5000

via ngrok/serveo/pagekite servers. Users can watch live stream videos on this web page and can also generate on-demand Timelapse for pictures clicked within a 1-hour time frame. The user has to select the start time of time-lapse in the Time-picker before hitting the ‘Time-lapse generate’ button. Users can access the live stream feature on multiple devices at the same time but should generate On-demand timelapse video only in any one device at a time. We have done all possible efforts to provide live streaming even when the user is generating a timelapse video.

Once the time-lapse is generated successfully, the user will get a link at bottom of the webpage to access the timelapse video by clicking on that link. If there are no pictures clicked for the time-lapse duration, a popup will be shown on the web page that "No Images found for the selected time period. Please enter different time".

User should not refresh the page, instead should select other valid time of continue to watch the live feed on the same page. Page refresh will resubmit the form and mary generate duplicate Time-lapse video.